Machinist

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

51-4041.00

RAPIDS CODE

0296

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| Tool Crib Learning names of raw materials and names and use of tools, jigs, fixtures and gauges. |  | 500 |
| Drills Power and radial drilling, tapping, ream-lapping, counterboring and countersinking, grinding drills, lubricants, cutting, speeds and feeds, safety. |  | 500 |
| Lathe-Engine Chucking, use of face plate, mandrel, steady rest and follow rest, centering, straight turning, facing taper, turning with taper attachment, offset tail stock and compound, drilling, reaming. |  | 1.5K |
| Milling Machine Plain, vertical and universal. Selection of cutters, methods of holding work, vise, clamps, diving head, circular table - plain or slab, milling, sawing, boring, fly cutter milling, vertical head, keyway cutting, slotting, spline milling, rack cutting, cutter milling, gear cutting, gang milling, form milling, speeds and feed, lubricants, safety. |  | 1K |
| Shaper and Planer Methods of holding work, vise, clamps, diving head, surface and angle cutting, keyway cutting, squaring, dovetailing, speeds and feeds, grinding tools, safety. |  | 500 |
| Surface Grinder Safety, selection of grinding wheels, speeds and feeds, mounting wheels, magnetic chuck, dressing wheels, plain or surface grinding, angle grinding, squaring. |  | 300 |
| Universal Grinder Safety, mounting wheels, speeds and feeds, dressing wheels, straight taper, angle face, form and hole grinding. |  | 500 |
| Cutter Grinder Safety, mounting wheels, setting up indexing attachments, clearance angles for various types of cutters and reamers, setting up for these angles, grinding plain, spiral and end mills, reamers, form cutters. |  | 600 |
| Heat Treatment Kinds of steel, S.A.E. classification, how to harden, draw, case and pack harden and anneal, use of pyrometer and color chart, hardness tests, quenching baths and safety. |  | 100 |
| Bench Work Filing, scraping, chipping, layout and assembly, use of gauge blocks and dial indicator, vernier height gauge, lapping, tapping and threading, lubricants, inspection, safety. |  | 500 |
| General Machinery Repair Inspection and adjusting, removing and replacing broken and worn parts of machine tools, scraping bearings and ways and rebuilding machines, welding. |  | 2K |
| Totals |  | OJT HRS 8K |

Machinist

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

51-4041.00

RAPIDS CODE

0296

| Related Instruction Online | RSI HRS |
| --- | --- |
| Safety | 50 |
| Mathematics General arithmetic processes to oblique trigonometry, compound angels and numerical control | 100 |
| Basic Blueprint Reading and Sketching Concepts of visualizing shapes, line usage on drawings, drawing and print production and basic title block format. | 100 |
| Tools and Equipment | 70 |
| Engine Lathes | 80 |
| Air Compressors | 4 |
| Drill Press | 4 |
| Grinding Technology Learn methods of hand and machine grinding. | 100 |
| Bench Layouts | 50 |
| V Block and Clamp | 13 |
| Jack Screw | 5 |
| Totals | RSI HRS 580 |