Plumber

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

47-2152.02

RAPIDS CODE

0432

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| Install plumbing or piping. Attach pipes to walls, structures, or fixtures, such as radiators or tanks, using brackets, clamps, tools, or welding equipment. Assemble pipe sections, tubing, or fittings, using couplings, clamps, screws, bolts, cement, plastic solvent, caulking, or soldering, brazing, or welding equipment. Install pipe assemblies, fittings, valves, appliances such as dishwashers or water heaters, or fixtures such as sinks or toilets, using hand or power tools. Anchor steel supports from ceiling joists to hold pipes in place. Install underground storm, sanitary, or water piping systems, extending piping as needed to connect fixtures and plumbing. Install pipe systems to support alternative energy-fueled systems, such as geothermal heating or cooling systems. |  | 1.3K |
| Maintain plumbing structures or fixtures. Modify, clean, or maintain pipe systems, units, fittings, or related machines or equipment, using hand or power tools. Shut off steam, water, or other gases or liquids from pipe sections, using valve keys or wrenches. Maintain or repair plumbing by replacing defective washers, replacing or mending broken pipes, or opening clogged drains. |  | 630 |
| Weld metal components. Attach pipes to walls, structures, or fixtures, such as radiators or tanks, using brackets, clamps, tools, or welding equipment. Assemble pipe sections, tubing, or fittings, using couplings, clamps, screws, bolts, cement, plastic solvent, caulking, or soldering, brazing, or welding equipment. Weld small pipes or special piping, using specialized techniques, equipment, or materials, such as computer-assisted welding or microchip fabrication. |  | 630 |
| Mark reference points on construction materials. Locate and mark the position of pipe installations, connections, passage holes, or fixtures in structures, using measuring instruments such as rulers or levels. |  | 210 |
| Measure materials or objects for installation or assembly. Locate and mark the position of pipe installations, connections, passage holes, or fixtures in structures, using measuring instruments such as rulers or levels. |  | 210 |
| Create construction or installation diagrams. Lay out full scale drawings of pipe systems, supports, or related equipment, according to blueprints. |  | 210 |
| Cut metal components for installation. Cut, thread, or hammer pipes to specifications, using tools such as saws, cutting torches, pipe threaders, or pipe benders. |  | 210 |
| Fabricate parts or components. Cut, thread, or hammer pipes to specifications, using tools such as saws, cutting torches, pipe threaders, or pipe benders. |  | 210 |
| Plan layout of construction, installation, or repairs. Plan pipe system layout, installation, or repair, according to specifications. |  | 210 |
| Inspect plumbing systems or fixtures. Inspect, examine, or test installed systems or pipe lines, using pressure gauge, hydrostatic testing, observation, or other methods. Fill pipes or plumbing fixtures with water or air and observe pressure gauges to detect and locate leaks. |  | 420 |
| Review blueprints or specifications to determine work requirements. Review blueprints, building codes, or specifications to determine work details or procedures. |  | 210 |
| Select construction materials. Select pipe sizes, types, or related materials, such as supports, hangers, or hydraulic cylinders, according to specifications. |  | 210 |
| Direct construction or extraction personnel. Direct helpers engaged in pipe cutting, preassembly, or installation of plumbing systems or components. |  | 210 |
| Clean equipment or facilities. Modify, clean, or maintain pipe systems, units, fittings, or related machines or equipment, using hand or power tools. |  | 210 |
| Estimate construction project costs. Estimate time, material, or labor costs for use in project plans. |  | 210 |
| Estimate construction project labor requirements. Estimate time, material, or labor costs for use in project plans. |  | 210 |
| Install gauges or controls. Install automatic controls to regulate pipe systems. |  | 210 |
| Record operational or environmental data. Keep records of work assignments. |  | 210 |
| Inspect work sites to determine condition or necessary repairs. Inspect structures to assess material or equipment needs, to establish the sequence of pipe installations, or to plan installation around obstructions, such as electrical wiring. |  | 210 |
| Repair worn, damaged, or defective mechanical parts. Repair or remove and replace system components. |  | 420 |
| Remove parts or components from equipment. Repair or remove and replace system components. |  | 210 |
| Replace worn, damaged, or defective mechanical parts. Repair or remove and replace system components. |  | 210 |
| Cut openings in existing structures. Cut openings in structures to accommodate pipes or pipe fittings, using hand or power tools. |  | 210 |
| Inspect work sites to identify potential environmental safety hazards. Inspect work sites for obstructions or holes that could cause structural weakness. |  | 210 |
| Install green plumbing or water handling systems. Install green plumbing equipment, such as faucet flow restrictors, dual-flush or pressure-assisted flush toilets, or tankless hot water heaters. Install fixtures, appliances, or equipment designed to reduce water or energy consumption. |  | 420 |
| Operate pumps or compressors. Operate motorized pumps to remove water from flooded manholes, basements, or facility floors. |  | 210 |
| Totals |  | OJT HRS 8K |

Plumber

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

47-2152.02

RAPIDS CODE

0432

| Related Instruction Online | RSI HRS |
| --- | --- |
| Mechanical Skills Related Instruction The 576 hours required to complete apprenticeship are broken down into 144 hours per year for a total of four years. The curriculum utilized for the instruction is the PHCC Plumbing Training Series, developed in partnership with the PHCC Educational Foundation Plumbing Apprentice and Journeyman Training Committee. A team of subject matter experts including contractors, teachers and code experts developed the curriculum to provide an industry-oriented approach. Writers with a background in the trade and teaching produced the content. The program is organized according to the U.S. Department of Labor Standards and is recognized by them to meet the qualifying standards for the classroom portion of apprentice programs. | 580 |
| Totals | RSI HRS 580 |