Energy Storage Technician

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

17-3023.00

RAPIDS CODE

0155(HY)

| Work Process Schedule | SKILLS |  |
| --- | --- | --- |
| Safety Demonstrates good housekeeping Demonstrates ladder safety Demonstrates the use of fall protection Demonstrates how to read an SDS (Safety Data Sheet) Demonstrates understanding and implementation of LOTO (Lockout/Tagout) program Ability to read one-line diagrams Deenergizing electrical equipment to isolate components Confirming electrical circuit has been deenergized Properly installs locks and tags Demonstrates how to read arc flash labels on electrical equipment/cabinets and identifies PPE required. Demonstrates how to mark off safety boundaries Demonstrates how to assess job safety hazards and fill out the JHA (Job Hazard Analysis) form. LION (Lithium Ion Battery) Shipping: Correctly Pack batteries for shipping Correctly Label for shipping Demonstrate knowledge of regulatory requirements for grounding Demonstrate knowledge of overhead line clearances PPE (Personal Protective Equipment) Use and Maintenance Properly inspect PPE Properly Wear PPE Properly clean PPE | 20 |  |
| Electricity Basics – Demonstrate the following tasks: Basic Electrical Troubleshooting Use a digital multimeter to verify voltage Use a digital multimeter to measure resistance Use a clamp on ammeter to measure current Wiring Basics Trace wire to identify circuit path and look for potential faults Read wire labels to help identify source location and destination of wiring circuit How to check continuity of CAN communications circuit Check insulation of wire/cable for any signs of visual damage (nicks in the wire, copper showing through) Check terminal connection points to verify they are making good contact, no loose connections Electrical Print Reading Identify the symbols, device numbers, and abbreviations for single line diagrams, elementary diagrams, and wiring diagrams Trace wiring according to a schematic to identify components of the circuit | 13 |  |
| Tools / Equipment – Explanation of functions and demonstration of utilization Digital Multimeter Identify the specifications and ranges of the multimeter to confirm it is acceptable to use for the device to be measured Use a digital multimeter to verify voltage Use a digital multimeter to measure resistance AC/DC Clamp Meter Use the clamp meter to measure the current flowing through a wire/cable. LCR (inductance (L), capacitance (C), and resistance (R)) Meter Use to measure an inductor CAN (Controller Area Network) Communications Device Identify the appropriate CAN device for the application and select necessary communication harness. Demonstrate ability to troubleshoot issue if device is not receiving/transmitting data as expected. Oscilloscope Battery pack Charging/Discharging Machine Megohmmeter (Insulation Resistance Tester) Battery Cell Balancing Machine Battery Air Tightness Detector Hygrometer – Use to measure humidity Torque wrench Thermal imaging camera | 19 |  |
| Environmental Impact Environmental Health / Impact Economics of Energy and Public Policy Recycling Decommissioning Renewable Energy Technology and Applications | 5 |  |
| Energy Storage System (ESS) Understanding Identify whether the Battery Energy Storage System (BESS) is using air-cooled or liquid-cooled battery packs. Be able to identify all components of the BESS and describe their functions: Battery rack, LV harness, HV DC Cables between the battery packs - to the control box - and to the DC Bus Combiner, DC Bus Combiner cabinet location, Rack grounding point, MBMU (Main Battery Management Unit), DC Disconnect Switch isolation to PCS, AC Cabinet Location – source of auxiliary power, Fire Suppression System cabinet and components, Chiller Demonstrate understanding of the Site organization of EMS and function (SCADA) - Understanding system controls and architecture, what is an integrator, asset owner, operator, OEM portion Identify which BMS (Battery Monitor System) software is needed to communicate with the BESS Identify location for connecting CAN device for communication to the BESS Connect USB CAN Device and connect to battery system using appropriate BMS Monitoring Software tool. Demonstrate understanding of displayed information Demonstrate understanding of displayed information and how to adjust settings according to site BESS configuration. Demonstrate ability to resolve any issues associated with lack of communication: Communication harness issue between CAN device and laptop Communication issue in the CAN circuit between battery racks or between battery rack and MBMU. | 9 |  |
| ESS Battery Fundamentals Identify major electrical components of a battery pack and describe their function: Battery cell, HV Busbar, Module connection busbar, Fan (on air-cooled pack), Coolant ports on liquid cooled pack), CSC, Positive and Negative battery terminals, Insulating film of battery cell, MSD. Use BMS Monitor software and demonstrate knowledge of the information displayed. Identify which battery cell is the highest and which is the lowest. Any issues noted? Are the rack voltages within expected tolerance (&gt;10mV Δ)? Demonstrate knowledge of the next steps for any issue identified in Step 6.2. | 5 |  |
| Electric Vehicle (EV) System Understanding Identify whether the EV is a Commercial Vehicle (CV) or Passenger Vehicle (PV) Be able to identify all components of the vehicle and describe their functions: Battery System Structure topology, Position distribution of components in vehicle Product structure: Battery pack components, Battery modules and cells, Power Distribution Unit (PDU), Thermal management system, Battery Management System (BMU), HV and LV harnesses Identify which BMS (Battery Monitor System) software is needed to communicate with the EV. Identify location for connecting CAN device for communication to the EV. Demonstrate the skill to follow the pin layout of the LV communications harness, identify the pins for 24Vdc power, key ON/wakeup signal, CAN H and CAN L and make a cable to communicate with the battery pack. Demonstrate ability to troubleshoot and resolve issues with CAN communications. | 7 |  |
| Commercial Vehicle (CV) Battery Fundamentals Identify major electrical components of a battery pack and describe their function: Top Cover and sealing gasket, HV Positive and Negative battery terminals, LV connector, MSD (Manual Service Disconnect), Balancing valve, Cell Supervisory Circuit (CSC), HV Busbar, Module connection busbar, Flexible Printed Circuit (FPC), Module heater, Coolant plate and coolant ports Connect to the CV battery pack system and demonstrate knowledge of the information displayed. Identify which battery cell is the highest and which is the lowest. Any issues noted? Demonstrate knowledge of the next steps for any issue identified in Step 8.2. | 4 |  |
| Battery Fundamentals – Passenger Vehicle (PV) Identify major electrical components of a battery pack and describe their function: Top Cover and sealing gasket, HV Positive and Negative battery terminals, LV connector, MSD (Manual Service Disconnect), Explosion-proof valve, DC Quick Charging connector, Coolant plate and coolant ports, S-Box (Sampling box) Connect to the CV battery pack system and demonstrate knowledge of the information displayed. Identify which battery cell is the highest and which is the lowest. Any issues noted? Demonstrate knowledge of the next steps for any issue identified in Step 9.2. | 4 |  |
| Maintenance Processes Project Planning Demonstrate project planning skills: Define Goals and Objectives, set success metrics, clarify stakeholders and roles, set budget parameters, set benchmarks and milestones for project deliverables, create timelines and schedules, set communication parameters General Battery Maintenance and Testing BESS Commissioning PV/CV Battery Inspection Process | 5 |  |
| Administrative Communication (Written/Oral) Grammar / Spelling Email Etiquette Professional language in an oral setting Texting and Group Chats Aptitude assessment Reports – GSS, LIFT, DR, JHA, PC, TRAVEL, LSR, Expense Report Written Final Report Card Sign Off – score of written final, assessment of OJL, classroom scores | 9 |  |
| Advanced Electrical Concepts PCS (Power Conversion System) Fundamentals Demonstrate understanding of one-line diagram for PCS to BESS and PCS to Utility Grid connection. Identify each component of the one-line diagram and describe their functions in the system. Demonstrate understanding of safety requirements and PPE for isolating PCS and for performing maintenance. Demonstrate steps for safely starting up PCS. Demonstrate steps to safely stop the PCS. Identify all the source of AC and DC power within the PCS and how to safely LOTO the system for maintenance. Identify the steps involved in performing 6-month maintenance. Identify the steps involved in performing Annual maintenance. | 9 |  |
| Decommission On-Site work Demonstrate understanding of key safety needs on a variety of Decommission project, including PPE Demonstrate understanding of equipment and needs on a variety of Decommission project Use understanding of electrical and battery concepts and equipment to successfully complete a variety of Decommission projects as part of a team Understand common customer needs on Decommission projects, and how to best communicate with customers in a variety of situations | 4 |  |
| Independent Work – ESS (Years 2-3) Demonstrate understanding of key safety needs on a variety of ESS projects, including commissioning and maintenance Demonstrate understanding of equipment and needs on a variety of ESS project, including commissioning and maintenance Use understanding of electrical and battery concepts and equipment to successfully complete a variety of ESS projects as part of a team, including commissioning and maintenance Understand common customer needs on ESS projects, and how to best communicate with customers in a variety of situations | 4 |  |
| Independent Work – EV (Years 2-3) Demonstrate understanding of key safety needs on a variety of EV projects, including on-site maintenance and troubleshooting Use understanding of electrical and battery concepts and equipment to successfully complete a variety of EV projects as part of a team, including commissioning and maintenance Understand common customer needs on EV projects, and how to best communicate with customers in a variety of situations | 3 |  |
| Independent Work – Logistics / Warehouse (Years 2-3) Demonstrate knowledge of key safety needs in a Battery Warehouse environment Assess, troubleshoot, and apply maintenance and electrical principals to maintain a supply of batteries in a warehouse environment Ensure safe, compliant unpacking and repacking of battery case using active client batteries Demonstrate knowledge of logistical compliance requirements and practical requirements to receive, unpack, store, package, and ship batteries across the country Understand common customer needs in Workshop and Logistic environments, and how to best communicate with customers in a variety of situations | 5 |  |
| Mentorship / Coaching – Journeyworker (Year 3&4) Shadow an existing senior team member in a mentorship position to develop a clear understanding of the requirements for mentoring Act as direct mentor to on-the-job apprentices while continuing to support project teams on ESS, EV, and rotations Regularly support the assessment and progress tracking of their mentee Perform at least one full “360 Performance Review” with themselves, their supervisor, and their mentee | 4 |  |
| Leadership Training – Senior Journeyworker (Year 4) Shadow existing Resource Manager to understand team needs beyond individual projects, including resourcing challenges. Participate in activities as appropriate. Shadow a Project Manager to understand project budgeting, scoping, and time management Shadow an Account Manager to understand client management, client communication, and the basics of customer sales on a variety of project | 3 |  |
| Totals | SKILLS 132 | OJT HRS 0 |

Energy Storage Technician

APPRENTICESHIP APPROACH

Hybrid

O*NET-SOC CODE

17-3023.00

RAPIDS CODE

0155(HY)

| Related Instruction Online | RSI HRS |
| --- | --- |
| Basic Electricity: understanding AC/DC theory | 24 |
| Basic Electric Troubleshooting | 32 |
| Wiring Basics | 16 |
| Insulation | 8 |
| Introduction to Low Voltage Circuit Breakers | 32 |
| Electrical Print Reading | 24 |
| Introduction to Protective Relays | 24 |
| Grounding and Bonding | 24 |
| Environmental Health / Impact | 8 |
| Economics of Energy and Public Safety | 8 |
| Recycling | 8 |
| Decommissioning | 8 |
| Renewable Energy Technology and Applications | 8 |
| Battery History | 8 |
| Battery Chemistry | 8 |
| Battery Types | 8 |
| Supply Chain Logistics | 8 |
| Battery Construction | 16 |
| Battery Uses | 8 |
| Pelican Case – Tool Kit Contents and Uses | 8 |
| Forklift/Trucks/Construction Equipment | 8 |
| Charging/Discharging Machine | 8 |
| Cell Balancing Machine | 8 |
| Insulation Resistance Testing with Megohmmeter | 8 |
| Battery Design | 8 |
| Battery Rack Design | 8 |
| Electric Vehicle Design | 8 |
| OSHA 10 for Construction and General Industry | 10 |
| NFPA70E Arc Flash Safety | 16 |
| US DOT Lithium Ion Battery Safety + Hazardous Material training | 8 |
| US DOT Lithium Ion Battery Shipping | 8 |
| First Aid | 16 |
| Electrical Safety for Industrial Facilities | 32 |
| Electrical Safety for Utilities | 32 |
| Electrical Safety for Inspectors | 32 |
| PPE (Personal Protective Equipment) Use and Maintenance | 8 |
| Cold Stress Training | 8 |
| Heat Illness Prevention | 8 |
| OSH Electrical Safety Related Work Practices | 16 |
| Air Tightness Testing | 16 |
| Torque Levels | 8 |
| Humidity Testing | 8 |
| Battery Capacity Testing | 16 |
| Polarity Checks for Battery Racks to DC Bus | 16 |
| Repair / Replace Fuses / Relays | 8 |
| Circuit Breaker Maintenance, Low Voltage | 40 |
| Circuit Breaker Maintenance, Medium Voltage | 32 |
| Circuit Breaker Maintenance, Molded and Insulated Case | 16 |
| Circuit Breaker Repair, SF6 | 32 |
| Software | 16 |
| Charging | 16 |
| Discharging | 16 |
| Updating Firmware / Software of the Equipment | 8 |
| Commissioning | 16 |
| Project Planning | 8 |
| LOTO Safety and Practices | 8 |
| Polarity Checks on Battery Racks for Commissioning | 8 |
| General Battery Maintenance and Testing | 32 |
| Communication (Written and Oral) | 16 |
| Reports | 8 |
| Aptitude Assessment | 16 |
| Written Final | 8 |
| Report Card Sign off | 8 |
| Inverter Training | 16 |
| Fiber optic Technician ¦ Certification Course | 36 |
| Line Safety ¦ Energized Overhead Contact Systems | 80 |
| National Electric Code 2023 and its Changes | 32 |
| OSHA Generation, Transmission, and Distribution | 16 |
| Power Quality and Harmonics | 32 |
| Solar Systems Photovoltaic Advance Certification | 32 |
| Substation Maintenance I | 40 |
| Substation Maintenance II | 40 |
| Transformer Maintenance and Testing | 40 |
| Advanced Transformer Maintenance and Testing | 40 |
| OEM ESS Level 1 Certification | 24 |
| OEM ESS Level 2 Certification | 24 |
| OEM CV Level 1 Certification | 24 |
| OEM CV Level 2 Certification | 24 |
| Totals | RSI HRS 1.4K |